iPerf Testing.

Requirements:

- Iperf Tool

- One AP (Set to channel depending on the interference requirement).

- Two test PC’s. (Laptops)

- Testing Device (DUT with proper setup)

Test Setup:

Test PC 1

As a server

Test PC 2

For DUT Logs

Connected through Ethernet

AP

Connected through Serial

DUT ( STA)

Steps to Perform:

- In your phone install app like WiFi analyser to check the interference of network in your test locations.

- Install the iperf tool in the Test PC 1.

- Disconnect the Network from both the Test PC’s (Turn on Airplane Mode).

- Connect an Ethernet cable from the router to the Test PC 1.

- Connect a serial cable from the device under test (DUT) to the Test PC 2 to retrieve DUT logs.

- To obtain the Ethernet IPV4 address in the Test PC 1, use the "ipconfig" command in the command prompt.

- Next,open Tera Term in the Test PC 2 to access the DUT logs.

- Device (DUT) should connect to the same AP using LUCI command in tera terminal.

Example:- “ LUCI_local 125 ASUS,Libre@123”

Here, ASUS is the AP SSID and Password is Libre@123.

- In the Test PC 2 in tera term, type the following command to check the connection:

Example: “ping 192.168.1.41” .

ping [insert the IP address of the Test PC 1]

- Now the user has to test the DUT both as server and client (i.e. tx or rx).

- Before beginning the test on the Test PC 2, make sure to check the RSSI value of the current network in Tera Terminal using the commands provided below. This will ensure accurate test results.

For LS10:

mlanutl wlan0 getsignal 1

For LS11:

wpa_cli scan
wpa_cli scan_results

For RX testing:

- While making the DUT as receiver give the following command ( Server Command ) in tera terminal in Test PC 2 to make as a server

iperf -s -u -i 1

- And from the Test PC 1 which will act as a Transmitter, go to the iperf tool path and open command prompt and give the below command (Client command) in terminal to make as a client.

iperf -c 192.168.1.132 -u -b 5M -i 1 -t 60

- Here need to replace it with DUT IP.

- “-u” for UDP test.

- “-b 5M” -b indicates bandwidth and based on test you can change the size.

- (Ex:-“ 5M,10M,15,…etc.) M - Mega bits per second

- “-t 60” -t indicates time and 60 is the total time period.

- Both (Server and Client) commands should be entered simultaneously on both Test PC’s.( Note that there are some connection issue to get rid of that  it is recommended to enter the client command first.)

- Once the test starts, you can observe the data transmission with the given bandwidth on both terminals. The test will run for 60 seconds and then stop. At the end of the test, you can view the average data transmission from 0 to 60 seconds.

- Once it is completed, you can run multiple iterations or try with different bandwidth and range to get average data.

For TX testing:

- While making the DUT as Transmitter give the following command (Client command) in tera terminal in Test PC 2 to make as a client.

iperf -c 192.168.1.132 -u -b 5M -i 1 -t 60

- Here need to replace it with Test PC 1 IP.

- Remaining everything same as mentioned in the above test.

- And from the Test PC 1 which will act as a receiver, give the below command ( Server Command ) from terminal (command prompt) to make as a server.

iperf -s -u -i 1

- Enter both the commands in both the Test PC’s then you can see the data transmission.

- Once the test is completed you can see the average data transmission and note it and start another test  as mentioned in the above test.

Sample Report:

| Mode of Connection | Antenna Type | Distance | RSSI | Bandwidth | Throughput test 1 (in Mbps) | Throughput test 2 (in Mbps) | Throughput test3 (in Mbps) | Average Throughput (in Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UDP | PCBA | 2mtr | -25 | 5M | 5.00 | 5.00 | 5.00 | 10.00 |
|  |  |  | -25 | 10M | 10.00 | 10.00 | 10.00 | 10.00 |
|  |  |  | -25 | 15M | 15.00 | 15.00 | 15.00 | 15.00 |
|  |  |  | -28 | 20M | 19.90 | 20.00 | 20.00 | 19.97 |